pfSense DoS Defense HomeLab

A cybersecurity homelab demonstrating network segmentation, Kali Linux DoS attack simulation on a Ubuntu VM, real-time packet capture, and firewall-based mitigation.

By Rushabh Roymon | GitHub Portfolio Project

| Attack Packets Generated 1,774,472 | Wireshark Capture 377,654+ packets |
| --- | --- |
| Attack Packet Rate 177,000 pps | Firewall Blocking Rate 100% |

Homelab Project Complete Documentation

# Project Objectives

- Stand up a pfSense firewall in VirtualBox with separate WAN (bridged) and LAN (internal) segments

- Put Kali Linux on the WAN side to simulate an external attacker

- Put Ubuntu Desktop on the LAN side as the victim host

- Demonstrate DoS traffic with hping3 and show it in Wireshark

- Mitigate the attack using pfSense firewall rules and view the resulting logs

# 2. Lab Topology & IP Plan

| Device / Segment | IP / Role |
| --- | --- |
| pfSense WAN | 192.168.29.241/24 (DHCP from router) |
| pfSense LAN | 192.168.1.1/24 (Static, DHCP Server) |
| Kali Linux (Attacker) | 192.168.29.7 (Bridged) |
| Ubuntu (Victim) | 192.168.1.100/24 (DHCP from pfSense) |
| Internal Network Name | intnet (VirtualBox Internal Network) |

Network Flow:

Kali (192.168.29.7) → pfSense WAN (192.168.29.241) → pfSense LAN (192.168.1.1) → Ubuntu (192.168.1.100)


┌───────────────────────────────────── ──────────────┐

Host Network (192.168.0.0/24)

Physical NIC: MediaTek Wi-Fi 7

└─────────────────────────────────────────────────────┘

▼

───────────┴──────────

|                                                          |

┌────▼───────┐            ┌────────▼──────┐

KALI                                                 PFSENSE

(Attacker)                                            (Firewall)

192.168.29.7                                   192.168.29.241      ← WAN (Bridged)

Bridged                                           192.168.1.1          ← LAN (Internal)

hping3                                            DHCP Server

└────────────┘             └───────┬───────┘

|

┌────── ▼────────┐

Internal Net

(intnet)

192.168.1.0/24

└───────────────┘

|

┌────────▼────────┐

UBUNTU

(Victim)

192.168.1.100/24

Wireshark

└─────────────────┘

# 3. Virtual Machine Setup

## 3.1 VM Specifications

| VM | Specs |
| --- | --- |
| pfSense | 2 GB RAM ¦ 2-Core CPU ¦ 15 GB Disk |
| Kali Linux | Pre-built VirtualBox Image (not ISO) |
| Ubuntu (Victim) | 2 GB RAM ¦ 2-Core CPU ¦ 20 GB Disk |
| Host OS | Windows 11 + VirtualBox 7.x |

## 3.2 Network Adapter Configuration on VirtualBox

pfSense Adapter 1 (WAN)

- Attached to: Bridged Adapter

- Physical NIC: MediaTek Wi-Fi 7

- Virtual Cable Connected: Enabled

pfSense Adapter 2 (LAN)

- Attached to: Internal Network

- Network Name: intnet

- Virtual Cable Connected: Enabled

Kali Adapter 1

- Attached to: Bridged Adapter

- Name: MediaTek Wi-Fi 7 MT7925 (same as pfSense WAN)

- Virtual Cable Connected

Ubuntu Adapter 1

- Attached to: Internal Network

- Name: intnet

- Virtual Cable Connected

4. pfSense Configuration

## 4.1 Interface Assignment

pfSense uses:

WAN = em0 (Bridged Adapter)

LAN = em1 (Internal Network - intnet)

pfSense Console

## 4.2 LAN Static IP Configuration

Console setup via Option 2 (Set interface IP address):

- IPv4 via DHCP? → n

- IPv4 Address → 192.168.1.1

- Subnet → 24

- Gateway → Enter (none - pfSense IS the gateway)

- Configure IPv6 address LAN interface via DHCP6 → n

- Enter the new LAN IPv6 address → Enter (none)

- Enable DHCP on LAN? → y

- DHCP Start: 192.168.1.100 | DHCP End: 192.168.1.199

- Revert to HTTP? → n (Keep the webConfigurator protocol on HTTPS)

pfSense will finish and display the LAN address (should be 192.168.1.1).

\

## 4.3 pfSense Console - Final Status

| WAN IP | 192.168.29.241 |
| --- | --- |
| LAN IP | 192.168.1.1/24 |
| DHCP Range | 192.168.1.100 – 192.168.1.199 |

## 4.4 Web GUI Access

Accessed pfSense Web GUI via WAN IP:

https://192.168.29.241

Username: admin | Password: pfsense

Note: Used pfctl -d to disable firewall temporarily in pfSense

5. IP & Routing Configuration

## 5.1 Ubuntu Network (Victim)

| IP Address | 192.168.1.100 |
| --- | --- |
| Gateway | 192.168.1.1 (pfSense) |
| DHCP Source | pfSense LAN DHCP |

## 5.2 Kali Network (Attacker)

| Kali (Attacker) IP Address | 192.168.29.7 |
| --- | --- |
| pfSense WAN IP Address | 192.168.29.241 |
| Target (Ubuntu) IP Address | 192.168.1.100 |

## 5.3 Static Route on Kali

Kali needs a route to reach the internal LAN (192.168.1.0/24) via pfSense WAN:

ip route add 192.168.1.0/24 via 192.168.29.241

Verified routing table after adding route:

default via 192.168.29.1 dev eth0 proto dhcp src 192.168.29.7 metric 100

192.168.1.0/24 via 192.168.29.241 dev eth0  ← Added

192.168.29.0/24 dev eth0 proto kernel scope link src 192.168.29.7 metric 100

Result: RTNETLINK answers: File exists

CONFIRMED via "ip route"

TEST KALI CONECTIVITY TO LAN:

From Kali:

bash

ping -c 3 192.168.1.100

100% packet loss - > the ping failed. The firewall rule isn't working yet or there's a routing issue.

Quick check on Kali, verify the static route is still there:

bash

ip route

Route is there: 192.168.1.0/24 via 192.168.29.241. The issue is pfSense isn't routing the traffic back.

Pinging from Ubuntu to Kali to see if it's one-way or both-way:

On Ubuntu:
bashping -c 3 192.168.29.7

Works (0% packet loss). The issue is Kali → Ubuntu is blocked.

This means pfSense is blocking return traffic. We need to add a rule on the LAN side to allow Ubuntu to respond back to Kali

Check Ubuntu UFW: If UFW is active, it could be blocking ICMP.

On Ubuntu:

bash

sudo ufw status

UFW is inactive, so it's not blocking.

To further diagnose the root cause

traceroute from Kali:

bash

traceroute 192.168.1.100

Traceroute is timing out at every hop, pfSense isn't responding or routing the traffic at all.

The issue: pfSense WAN firewall rule is blocking ICMP by default.

## 6. pfSense Firewall (Web GUI)

## 6.1 Enable DHCP Server on LAN

- Services → DHCP Server → LAN

- Range: 192.168.1.100 to 192.168.1.199

Ubuntu got 192.168.1.100 from it automatically. pfSense doesn't need DHCP from anything, it has a static IP on the LAN side (192.168.1.1). On the WAN side, pfSense gets DHCP from your home router (192.168.29.241).

## pfSense Firewall Rules

## 6.2 Allow Rule: Kali → Ubuntu

Added to Firewall ▸ Rules ▸ WAN to allow the initial attack traffic:

| Setting | Value |
| --- | --- |
| Action | Pass |
| Interface | WAN |
| Address Family | IPv4 |
| Protocol | Any |
| Source | 192.168.29.7 (Kali) |
| Destination | 192.168.1.100 (Ubuntu) |
| Description | Allow Kali to Ubuntu |

## 6.3 Connectivity Verification

After adding the allow rule 

Ping works now -> 0% packet loss

7. DoS Attack Demonstration

## 7.1 Wireshark Setup on Ubuntu

Installed and launched Wireshark on Ubuntu for real-time packet capture:

sudo apt install -y wireshark

sudo wireshark &

Selected interface: enp0s3 → Double-clicked to start capture

- Status: "<live capture in progress>"

- Packet table is empty (waiting for traffic)

## 7.2 ICMP Flood Attack from Kali

Launched hping3 ICMP flood attack targeting Ubuntu:

sudo hping3 -1 --flood 192.168.1.100

Attack output:

HPING 192.168.1.100 (eth0 192.168.1.100): icmp mode set, 28 headers + 0 data bytes

hping in flood mode, no replies will be shown

Wireshark capture evidence:

- 377,654+ packets captured in real-time

- Source: 192.168.29.7 (Kali attacker)

- Destination: 192.168.1.100 (Ubuntu victim)

- Protocol: ICMP Echo Request/Reply

- "<live capture in progress>" confirmed active attack

Stop the hping3 flood on Kali

Results:

- Total packets transmitted: 1,774,472

- Packets received: 0 (flood mode - expected)

- Attack duration: ~10 seconds

8. Attack Mitigation & Blocking

## 8.1 Creating the Block Rule

Added blocking rule at TOP of WAN ruleset (above the allow rule):

| Setting | Value |
| --- | --- |
| Action | Block |
| Interface | WAN |
| Address Family | IPv4 |
| Protocol | Any |
| Source | 192.168.29.7 (Kali) |
| Destination | 192.168.1.100/24 (Ubuntu) |
| Logging | Enabled ✔ |
| Description | Block Kali DoS |

## 8.2 Rule Precedence

With the block rule at TOP, pfSense evaluates in this order:

| Priority | Action | Protocol | Source | Description |
| --- | --- | --- | --- | --- |
| ❌ #1 | BLOCK | IPv4 * | 192.168.29.7 → 192.168.1.100 | Block Kali DoS ← STOPS HERE |
| ✔ #2 | PASS | IPv4 * | 192.168.29.7 → 192.168.1.100 | Allow Kali to Ubuntu (never reached) |
| ✔ #3 | PASS (Default) | TCP | Home Network → pfSense | Allow GUI Access |

Figure 21: pfSense - All three rules applied (Block on top, Allow below)

## 8.3 Firewall Logs

Status ▸ System Logs ▸ Firewall ▸ Normal View showed:

- Thousands of blocked ICMP packets blocked

- Logging confirmed attack source, destination, and protocol

- Timestamps verified attack timeline

- Rule: Block Kali DoS is not seen (Identified Logging was not Enabled)

## 8.4 Logging Enabled on Block Rule

Edited the block rule to enable logging for forensic evidence:

- Firewall ▸ Rules ▸ WAN ▸ Edit (Block Kali DoS rule)

- Scrolled to Extra Options

- Checked: ☑ Log packets that are handled by this rule

- Saved → Applied Changes

9. Verification & Results

| Test | Before Block Rule | After Block Rule |
| --- | --- | --- |
| ping -c 3 192.168.1.100 | 0% packet loss (attack passing) | 100% packet loss (attack BLOCKED) |
| Wireshark on Ubuntu | 377,654+ ICMP packets flooding in | Zero new attack packets from Kali |
| pfSense Logs | No block logs (rule not yet added) | Block Kali DoS entries logged |

RED = Attack phase (before block rule): packets reaching Ubuntu

GREEN = Mitigated phase (after block rule): 100% packet loss = attack blocked

10. Technical Learnings & Key Takeaways

| Learning | Detail |
| --- | --- |
| Firewall Rule Precedence | Block rule MUST be above allow rule. pfSense evaluates top-down, stops at first match. |
| Address Family | IPv4 addresses require IPv4-only rules. IPv4+IPv6 caused errors with specific IPs. |
| DHCP Server Requirement | LAN interface must have static IP before DHCP server can be enabled. |
| Configuring LAN adapter on pfSense | LAN adapter had to be enabled and configured on the pfSense interface also disabling IPv6. |
| pfSense Web GUI Access | Use pfctl -d to disable firewall for initial GUI access |
| Wireshark Interface | Run sudo wireshark & (not just wireshark) to capture as root on Ubuntu and selecting the correct adapter to capture is key |
| Logging for Forensics | Always enable logging on security rules to create forensic evidence of attacks. |

# 11. Skills Demonstrated

| Category | Skills |
| --- | --- |
| Firewall Admin | pfSense rule creation, rule precedence, stateful filtering, logging, DHCP |
| Network Architecture | Multi-zone segmentation, WAN/LAN design, DHCP, DNS, routing |
| Attack Simulation | hping3 ICMP flood, traffic generation, attack verification |
| Traffic Analysis | Wireshark live capture, packet forensics, protocol analysis |
| Stateful Firewall Behavior | pfSense is a stateful firewall. Return traffic from allowed connections is automatically permitted without explicit rules. |
| Linux Admin | Kali & Ubuntu setup, static routing, UFW, CLI networking tools |
| Virtualization | VirtualBox VM management, Bridged/Internal adapters, network design |
| Incident Response | Threat identification, rapid rule deployment, blocking verification |

# Summary

This lab successfully demonstrates a complete DoS defense workflow:

- pfSense firewall with WAN/LAN segmentation

- Kali Linux simulating external attacker

- Ubuntu Desktop as victim on isolated LAN

- 1.7M+ packet ICMP flood attack demonstrated

- 377,654+ packets captured with Wireshark

- Firewall block rule achieving 100% attack mitigation

- Logging and forensic evidence